State Highway and Transportation Department

Concrete Plant Inspector’s CheckList

1.	Aggregates

- Stockpile foundation condition

- Aggregate stockpiled properly

- Handling of aggregates

- Cement and Fly Ash

- Has material been properly sampled?

- Are storage facilities adequate?

- Have proper certificates been received?

- Batching Equipment

- Bins and loading hopper checked for conditions and compliance with specifications

- All scales checked and calibrated

- Sensitivity of scales checked

- Water meter checked for accuracy

- Transit mix truck condition

- Revolution counter operation

- Mix Proportions

- Approved mix design received

- Size and batch determined

- Mix design proportions adjusted for desired air content

- Batch Quantities

- Dry weights per batch established for:

- Sacks or pounds of cement and fly ash

- Pounds of fine aggregate

- Pounds of coarse aggregate

- Total water required per batch

- Total yield (including air) per batch

- Quantity of air entraining agent required per batch

- Quantity of other additives required per batch

- Moisture Content

- Percent total moisture for each aggregate

- Gallons of effective moisture per batch

- Gallons of water to be added at mixer

- Scale Weights per Batch

- Cement same as dry weight

- Fly ash weight

- Aggregate (dry weight plus total moisture in pounds per each aggregate)

- Scales balanced and counterbalances securely fastened

- Scale weights for each material set and checked

- Admixture measurement devices adjusted to measure correct amount

- Water meter adjusted

- Batching and Hauling

- Batch trucks adequate for size of batch

- No spillage or contamination while loading

- Empty batch trucks clean and free of foreign material

- Condition of agitating and non-agitating equipment for hauling Central Mix Concrete

- Pre-wet mixer at beginning of operations

- Set revolution counter to zero at beginning of operations

- Routine Duties

- Systematic check of scale settings during the day

- Regular daily checks on cement received and used

- Constant testing and checking moisture content of aggregates

- Daily report prepared and records kept up to date

- Fill out haul tickets