State Highway and Transportation Department

Concrete Inspector’s Check List

Site of Delivery

- Assemble all test equipment

- Thermometers

- Air meter

- Scales

- Slump Cone

- Buckets

- Cylinder molds

- Measuring Tape

- Wheel Barrow

- Obtain Haul Tickets

- Check revolution count

- Check total water added at plant

- Check time of beginning of mixing

- Check Mix Consistency

- Allow contractor to add additional water if slump of mix is to low provided maximum water to cement ratio is not exceeded

- Mix additional 30 revolutions is water is added at site

- Allow discharge when required mixing revolutions have been completed

- Test air content

- Determine unit weight

- Test slump

- Check mix temperature

- Check atmospheric temperature

- Mold and cure concrete cylinders for strength tests

- Monitor time from introduction of cement at mixer to time of discharge ( 1 hour max.)

- Complete haul ticket data

- Fill out daily report