State Transportation Department

Bridge Deck

Re-bar CheckList

- Before Placement

- Obtain certificates of compliance

- Check bundles of steel for proper tags and identification

- Check for specification requirements as to grade, size, type, coating, bending, length, supplementary requirements, etc.

- Check coated rebar for handling and transportation damage

- Assure damaged areas are coated prior to use

- Assure metal chairs and other rebar support accessories meet requirements

- Assure that tie wire and support chairs for coated rebar are plastic coated

- Check for proper storage of rebar

- Placement

- Check size, location and spacing against plans

- Check rebar cover

- Check adequacy of ties and tie downs

- Check type and adequacy of support system

- Check clearance from forms and cover depth

- Check that forms are clean and free from oil, dirt, sawdust, etc.

- Check splice locations against plans

- Recheck coated rebar for coating damage.  Patch as required.

- Record placement in daily report

- During Concrete Placement

- Assure that rebar is not displaced during concrete placement

- Assure that rebar is not excessively vibrated

- Spot check rebar cover on bridge decks behind finishing machine