Traffic Control Guidelines (Emphasis Points)

| PIN _____________ | Project: ___________________________________________________ Location: ___________________________________________________ |
| --- | --- |

- Sign Condition and MUTCD Consistency -

- Unneeded Signs -

- Low-Mounted Signs -

- Advance Warning Signs -

- Sign Visibility -

- Flagger Signs -

- Flagger Control -

- Pedestrian and Worker Safety -

- Merge Taper Lengths -

- Temporary Concrete Barrier -

- Pavement Bumps -

- Temporary Sign Supports -

- Channelization Devices -

| Inspected by: _______________ Area Engineer | Date: ________________ |
| --- | --- |

MP&T Emphasis Points Guidance

- Sign Condition and MUTCD Consistency

  - Refer to Quality Standards for Work Zone Traffic Control Devices for acceptable condition levels.

  - Faded and deteriorated panels not acceptable.

  - Reflectorization in good condition and clean.

  - Non-standard legends not acceptable

  - Non-standard colors not acceptable

  - Signs look as they appear in MUTCD §6F, see attached pages 6F-1 to 6F-2 and 6F14 to 6F20.

- Unneeded Signs

  - Fully covered, removed, or otherwise completely eliminated from driver's view.

  - Flagger signs covered/removed when flagger is not present.

- Low-Mounted Signs

  - No low mounted signs permitted, except for flexible panels meeting current specifications.

  - Adhere to MUTCD §6F, for details see attached pages 6F4 and 6F6.

- Advance Warning

  - No overlapping, conflicting or unneeded advance warning signs.

  - Refer to MUTCD Appendix 6H for layout reference details, see attached various pages 6H4 to 6H79 for typical approach diagram layouts.

- Sign Visibility

  - Not hidden by foliage, roadway geometry or other signs.

  - Approach sight distance is adequate.

- Flagger Signs

  - Used only when flagger is present and visible to the motorist. They must be covered or removed at all other times.

  - Adhere to attached MUTCD page 6E-2.

- Flagger Control

  - Stop/Slow paddles are to be used where appropriate.

  - Flaggers must be positioned to provide adequate visibility and to enhance the safety of the flaggers.

  - Adhere to MUTCD §6E, see attached pages 6E3 and 6E4.

  - Refer to __DOT EI 93-022, Flagger Station Enhanced Setups for further guidance.

- Pedestrian and Worker Safety

  - Reasonably smooth, continuous, safe and convenient travel way provided and maintained for anticipated bicycle and pedestrian traffic at all times.

  - Adhere to MUTCD §6D, see attached pages 6D-1 to 6D-5.

  - Refer to State spec for further guidance.

- Merge Taper Lengths

  - Location of tapers should provide optimal visibility to approaching motorists.

  - Meet minimum requirements in MUTCD §6C-08, see attached pages 6C-9 to 6C-11.

  - Refer to State Spec for further guidance.

- Temporary Concrete Barrier

  - End flared minimum of 12 feet away from pavement on high-speed roads, if not flared, must be protected by impact attenuator device.

  - Barrier-flares preceded by channelizing devices and buffer spaces.

  - Barrier runs designed to fully protect the hazard.

  - Adhere to State spec

- Pavement Bumps

  - Reasonably smooth riding surface maintained.

  - Transverse pavement joints and paving rebates must be treated with temporary shims (60:1 taper).

  - "Bump Ahead" signs posted upstream on approach.

  - Channelizing device or object marker placed at bump.

- Temporary Sign Supports

  - All temporary supports not protected by guiderail must meet requirements of State Standard Specifications.

  - Type A metal supports must meet State Standard Material Specifications

  - Stub heights must not exceed 100 mm (4").

- Channelizing Devices

  - Good physical condition.

  - Placed at outer limits of the roadway available for use by vehicles.

  - Adhere to MUTCD Chapter 6F see attached pages 6F41 to 6F43.

  - Refer to State Spec for further guidance.

MP&T Emphasis Points Reference

- Selected FHWA Millennium MUTCD Sections

  - Chapter 6C, Temporary Traffic Control Elements

Traffic Control Plans, Traffic Control Zones, Components of Work Zones, Tapers, Detours and Diversions, Pilot Car

  - Chapter 6D, Pedestrian and Worker Safety

Pedestrian Considerations, Worker Considerations

  - Chapter 6E, Flagger Control

Flagger Qualifications, High-Visibility Clothing, Hand-Signaling Devices, Flagger Procedures, Flagger Stations

  - Chapter 6F, Temporary Traffic Control Zone Devices

Types of Devices; General Sign Characteristics; Sign Placement; Sign Maintenance; Warning Sign Function, Design, and Application; Channelizing Devices; Cones; Tubular Markers

  - Chapter 6H, Typical Work Zone Applications

Distance Between Advance Warning Signs, TA-1 Work Beyond the Shoulder, TA-10 Lane Closure on Two-Lane Road Using Flaggers, TA-11 Lane Closure on Low-Volume Two-Lane Road, TA-11 Lane Closure on Two-Lane Road Using Traffic Control Signals, TA-35 Mobile Operation on Multilane Road, TA-36 Lane Shift on Freeway, TA-37 Double Lane Closure on Freeway, TA-38 Interior Lane Closure on Freeway, TA-45 Temporary Reversible Lane Using Movable Barriers

- State DOT Engineering Instructions, Bulletins and Directives

List references

- Miscellaneous State DOT Reference Material

List references

Bottom of Form